STATEMENT OF WORK

GAOA Star Meadows MEP, Repairs and Utility Installations

Tally Lake Ranger District, Flathead National Forest

SECTION C – DESCRIPTION

# C.1   PROJECT SCOPE

- Description of Work: Work includes complete operational assembly of the following items.

- Hydronic Heating:  - HVAC. This project requires submittal of specialized design and construction of heating system features in buildings. The work covered includes piping, pumps, and associated components used in boilers, hydronic coils, and radiant floor heating systems.

- Air Heating and Distribution: HVAC shall unify hydronic heat and air distribution by installing units, grills and ducts at appropriate locales, sized for quick response to heating and fan cooling demand. Air handling work includes sensors, temperature controllers, wire, hardware, brackets, valves, pipes, fitting, and filters.

- Propane Service:  Install a concrete pad and propane yard-lines between a government supplied tank and appliances. Work includes all trenching, piping, regulator(s), shut offs, and the necessary code and manufacturer appurtenances required for complete and operational fuel delivery system. Interior plumbing for fuel supply to a boiler/on-demand water heater and new fireplaces.

- Propane Fireplaces: Install two new propane fireplaces to include all ductwork, venting, floor hearth, and wall shields.

- Water Supply:  Retrofit the existing well to provide a complete pressurized water supply to the dwelling with functional distribution to a kitchen faucet.  All appurtenance needed to complete a  water service connection, including well casing pitless adaptor, installing a well pump, a water supply pipeline, and electrical pump controls are included work.

- Sewer Drain: Stub in new sewer drains for a toilet, shower, and vanity sink from the existing septic tank to the basement bathroom and maintain a connection to the kitchen sink.

- Miscellaneous: Utility locates, permits, demolition, excavation, and all other work shown in the plans, specifications, and schedule of Base and Option line items are incidental to this work.

- Project Location:	Star Meadows Rental Cabin

5250 Star Meadow Rd

Whitefish, MT 59937

- Period of Performance: Estimated to be from March 2027 through June 29, 2027

- Technical Contact:	Patrick Siers – Project COR

(406)758-5347

patrick.siers@usda.gov

C.3     GENERAL SPECIFICATIONS

- Existing Facilities.  All existing facility structures damaged by the Contractor shall be replaced or repaired by the Contractor at no additional cost to the Government.  This includes any facilities inside or outside the project limits, including along any access routes.

Do not trim or damage vegetation except with specific Contracting Officers (CO) approval.

- Pollution and Erosion Control.  During all operations under this contract, the Contractor shall exercise reasonable measures to prevent or minimize soil erosion and pollution of water and other resources.  All refuse, including garbage, rubbish, and solid or liquid wastes, from the Contractor's operations, including camps, parking areas, and equipment maintenance areas shall be stored, collected, and disposed of in a sanitary and nuisance-free manner.  Refuse shall be disposed of in State-approved landfills unless otherwise approved by the Contracting Officer (CO).

c)  Coordination with District Operations.  The work will take place in areas critical to daily Ranger District operations.  The Contractor shall maintain regular contact and coordination with the district representative to assure that district operations are not hindered by the work.

d)  Storage and Stockpiling.  The Contractor shall confine storage of equipment, materials, and vehicles to within the limits delineated by the Contracting Officer. The Contractor shall be solely responsible for the loss of or damage to materials stored on Forest Service lands or elsewhere.

e)  Coordination with Other Contractors.  The Contractor shall maintain regular contact and coordination with all other contractors working on the district compounds to assure that the operations of other contractors are not hindered by the work.

f)  Worksite Cleanliness and Final Cleanup.  Maintain the project work areas in a clean and orderly condition throughout the progress of work under this contract.  All refuse, including garbage, rubbish, and solid or liquid wastes from the Contractor's operations shall be stored and collected in a sanitary manner and disposed of off Government property in a legal manner.  Maintain strict control of hazardous materials to prevent contamination of the site.

g)  Equipment Specifications.  Equipment and tools shall be in good mechanical and operating condition, fully capable of performing all phases of the specified work.

h)  Weather Conditions.  Work will be suspended if, in the opinion of the Contracting Officer, weather conditions preclude effective protection of resources, site rehabilitation, or otherwise impact project end results in a significant, negative manner.

i)  Sanitary Facilities – Vault Toilet and Handpump are available on site.  Water supply will be disrupted as part of construction activities.

j)  Notification.  Contractor shall notify the C.O. two weeks in advance of entering work site.

#### C-4  HAULING AND DISPOSAL OF MATERIALS

a)  All loads shall be completely covered during hauling.

b)  All materials removed from the site shall be disposed of in a legal manner off National Forest System lands. All fees associated with material disposal shall be paid by the Contractor.

## C-5  CLEANUP AND DISPOSAL

#### The Contractor shall always keep the area in an orderly condition. Upon completing the work, the Contractor shall remove from the site and premises any rubbish, tools, equipment, and materials that are not the property of the Forest Service.  Upon completing the work, the Contractor shall leave the work areas in a clean, neat, and orderly condition satisfactory to the Contracting Officer.

## C-6  EXISTING UTILITIES

The Contractor is responsible for location of all utilities prior to beginning work. Damaged utilities shall be repaired at the Contractor’s expense.

C-7 - FOOD AND GARBAGE STORAGE

All food, beverages, and scented or flavored toiletries must be stored unavailable to bears at night and when unattended during the day.  Attractants include but are not limited to canned food, pop, beer, garbage, grease, processed livestock or pet food, toothpaste, and chap stick.  All attractants shall be hung at least 10-feet off the ground and 4-feet from any vertical support, or stored inside a bear resistant container or vehicle.  Cars, pick-up cabs, enclosed horse trailers, hard sided campers or camper shells are all considered bear resistant.  Food should be stored out of sight in vehicles.  Coolers, pop-up campers, backpacks, and tents are not considered bear resistant.

#### C-8 – FIRE CONTROL  (FS.236.1)

- Contractor's Responsibility for Contractor-Caused Fires – The Contractor, whether or not directed by the Forest Service, shall immediately extinguish, without expense to the Government, all fires on or in the vicinity of the project which are caused by Contractor's employees, whether set directly or indirectly as a result of Contractor operations.  The Contractor may be held liable for all damages and costs of additional labor, subsistence, equipment, supplies, and transportation resulting from fires set or caused by the Contractor's employees or resulting from contract operations.

- Other Fires – For the purpose of fighting forest fires on or in the vicinity of the project, which are not caused by the Contractor or his employees, the Contractor when requested by the Contracting Officer shall place his employees and equipment temporarily at the disposal of the Forest Service. Payment for such services will be made by the Government at not less than the current rate for fire-fighting services established by the Forest Service in the area concerned.

Any employees and equipment furnished will be relieved from fire fighting as soon as the Forest Service finds that it is practicable to employ other labor and equipment adequate for the protection of the area.

An equitable adjustment in contract time may be made for this period.

- Fire Plan – At all times during closed fire season period, as specified by State law, the Contractor shall comply with each of the following provisions to the extent applicable to his operation under the contract.

  - Fire Tools – The Contractor will provide for each employee in the contract area at least one approved hand tool of a type appropriate in the contract area, such as shovel, pulaski, or ax.  Tools required and furnished under (2) and (4) below, shall count toward fulfillment of the above requirement.  Where additional tools, beyond those required under (2) and (4) below, are to be provided, the Contractor shall seal such tools in one or more boxes painted red and marked "Tools for Fire Only."  All tools required herein shall be kept sharp and in good serviceable condition and maintained at locations to be designated by the Forest Service.

  - Fire Extinguishers and Tools on Mobile or Stationary Equipment – Each unit of powered equipment used in connection with this contract, including automobiles, trucks, tractors, etc., shall be equipped with serviceable tools and fire extinguishers as follows:

One - fire extinguisher, dry chemical type of not less than 2-1/2 pound capacity with 4 BC or higher rating.

One - shovel, round point #0 lady or equal.

One - ax, 2 pounds or over, 26-inch minimum length, or one pulaski.

One - water container (at least 1-gallon capacity), not required with stationary equipment.

  - Blasting – Fuse or prima cord shall not be used unless authorized in writing by the CO with special precautions stated.

  - Smoking – Smoking shall not be permitted within the contract area except on surfaced or dirt roads, at landings, within closed vehicles, in camps, or at other posted places, and shall never be allowed while working or traveling on foot.

  - Storage of Petroleum and Other Highly Flammable Products – Gasoline, oil, grease, or other highly flammable material will be stored either in a separate building used exclusively for such storage, or at a site where all combustible debris and vegetation is cleared away within a radius of 25 feet. Fire extinguishers and/or sand barrels may be required at such locations specified by the Forest Service when unusually hazardous conditions exist.

  - Precautions for Stoves – Stovepipes on all temporary buildings, trailers, and tents using wood burning stoves, will be equipped with roof jacks and serviceable spark arresters of mesh with openings no larger than 5/8 inch.

All stovepipes, inside and out, will not be closer than 2 feet from any wood or other flammable material or 1 foot if the combustible material is protected by a metal or asbestos shield.

  - Welding – Welding or use of cutting torches will be permitted only in areas that have been cleared or are free of all material capable of carrying fire.  Flammable debris and vegetation must be removed from within a minimum of 10 feet radius of all welding and cutting torch operations.  A shovel and a 5-gallon standard backpack water container (filled), with hand pump attached, shall be immediately available for use in the event of a fire start.

  - Fire Plan – Prior to initiating work on the contract area, during the closed fire season period, a fire prevention and suppression plan will usually be prepared.  The Contractor and the Forest Service will jointly prepare this plan.  Such plan shall include a detailed list of men and equipment at the Contractor's disposal for implementing the plan.  The fire plan shall also specify additional measures and/or special requirements, such as Hoot Owl restrictions, necessary during periods of critical fire weather conditions.

When Hoot Owl restrictions are invoked, the Forest Service may curtail or shut down all or portions of a Contractor's operations.  The following requirements may be imposed:

- All high fire risk operations could be terminated at 1300 local time.

- All burning could be stopped, including debris burning fires.

- Patrolman may be required for a period of 2 hours after high fire risk operations and 1 hour after end of work shift.  The Contractor shall provide the patrolman unless the contract is for equipment rental, in which case the Government will provide the patrolman.

# C-9  MEASUREMENT AND PAYMENT

The method of measurement will be as designated in the SCHEDULE OF ITEMS.  The accepted quantity will be paid for at the contract unit price for the Pay Item shown in the SCHEDULE OF ITEMS.  Payment will be for all labor, equipment, transportation, supervision, operating supplies, materials, and incidentals necessary to complete the work as specified.

SECTION G--CONTRACT ADMINISTRATION DATA

### GOVERNMENT-FURNISHED PROPERTY

There is no government property provided for in his contract.

#### H-1 NOXIOUS WEED CONTROL

Prior to mobilizing onto National Forest lands, roads, or trails, the contractor shall sufficiently clean any equipment by removing all visible dirt and plant materials.  All subsequent move-outs/move-ins by the same piece of equipment shall be equally cleaned.

Prior to any equipment move-in, the Contractor shall make the equipment available for a Forest Service inspection at an agreed location.

#### H-2  SECURITY AND PROTECTION

- Security and Protection – Security is the Contractor’s responsibility.  Limit access to the construction area to persons involved in the work.  Provide secure storage for materials that are stored on site.  Secure completed work as required to prevent loss.

Barricades, warning signs and light shall comply with standards and code requirements for erection of structurally adequate barricades.  All road warning signs, barricades, and traffic control devices shall conform to MUTCD.  Paint with appropriate colors, graphics, and warning signs to inform personnel and the public of the hazard being protected against.  Where appropriate and needed, provide lighting, including flashing red or amber lights.

- Maintenance of Traffic – The Contractor is authorized to use roads under the jurisdiction of the Forest Service, and open to public travel, for all activities necessary to complete this contract, subject to limitations and authorizations shown on the drawings or specified under this contract when such use will not damage the roads or national forest resources and when traffic can be accommodated safely.

Except as allowed in the contract, trailheads and roads shall remain open throughout the construction project.  Perform no work that interferes or conflicts with traffic or existing access to the roadway surface until a plan for satisfactorily handling the traffic has been approved.

Before shutting down operations, take all necessary precautions to prevent damage to the project, such as temporary detours, approaches, crossings, or intersections; and provide for normal drainage and minimization of erosion.  Leave all travel ways in a condition suitable for traffic.

- Protection of Improvements – The Contractor shall complete sequences of construction activity in order to prevent damage to equipment, material, and articles incorporated into the work covered by this contract.